# PCleaner

Final Report

Ahmed Amr

Philopatear Salib

Nourhan Ayman

MCT 382: Design of MCT (2) | 22/5/2019

Table of Contents

Table of Figures	4

Introduction	6

Problem Statement	6

Motivation	6

Project Objectives	6

Design Thinking and Market Research	6

Customer Segment	6

Primary Features	6

Secondary Features	6

Value Proposition Canvas	7

Lean CAnavs	8

MarkeT Rsearch Survey	9

Product concept	9

Analyzed Data	10

Product Design and Human–Machine Interface	11

Ergonomics and usability	11

Ready to Market	12

Form Follows Function	12

Realistic Render	13

Design Development	14

Conceptual Design	14

Design Alternatives	14

Propsed Design	15

CAD Modelling	15

Mechanical DEsign	15

Actuator Sizing	17

MATLab Simulation	17

Simulink (Simscape Model – Velocity Control)	17

Results	19

Simplified Model	19

Results	20

Hand Calculaiotns	21

Horizontal Forces	21

Selected Pump/Actuator	1

VerTical Forces	1

Path Generation	1

Simulink Model	2

System Interconnection Diagram	4

Electrical Schematics	5

Circuit Schematics	5

Circuit Layouts	6

Power Sizing	8

Software Architecture	9

Project Plan	10

Project Timeline	11

Manufacturing Timeline	11

Integration Plan	12

Componenets Integration	12

Modular Integration	12

Saptial Integration	12

Test Plan	13

System Verification	13

System Validation	13

Bill of Materials and Cost	13

Conclusion	14

Time Line……………………………………………………………………………………………………………………………………………………..………20

Conclusion………………………………….……………………………………………………………………………………………………………….20

# Table of Figures

Figure 1: Value Proposition Canvas	6

Figure 2: Lean Canvas	7

Figure 3: Market Research Survey	8

Figure 4: Analyzed Data if customers would buy the product or not	9

Figure 5: Analyzed Data if customers have bought such a product before	9

Figure 6: Analyzed Data of how often would customers use this product.	9

Figure 7: Analyzed Data of how much would customers pay for such a product.	9

Figure 8: Analyzed Data of how many males or females would use this product.	9

Figure 9: Analyzed data of the ranges of ages whom would use this product.	10

Figure 10: Handle facilitates moving of robot	10

Figure 11:  Proposed Design	14

Figure 12: Overall Model – Velocity Control	17

Figure 13: System	18

Figure 14: DC Motor & Driver	18

Figure 15: Load & sensing	18

Figure 16: Pump flow rate	   Figure 17: Vehicle Velocity	19

Figure 18: Simplified Model	19

Figure 19: Vehicle Velocity (Simplified Model)	 ……………………………….…………………... ……………………………………….20

Figure 20: Thrust acting on vehicle (Simplified Model)	20

Figure 21: Path Generation	2

Figure 22: Thrusterx	2

Figure 23: Drag Forcex	3

Figure 24: Angle Scope	3

Figure 25: Displacement Scope	3

Figure 26: Turning a bilge pump into a thruster	5

Figure 27: SID	6

Figure 28: 24 AWG	7

Figure 29: ATmega328 Stand Alone Schematic Circuit	7

Figure 30:  (a) ATmega328PU Stand Alone Layout Circuit	8

Figure 31:  Integrated Circuit Schematic	8

Figure 32: Path of Vehicle along a pool	10

Figure 33: Flowchart	10

Figure 34: VDI 2206	11

Figure 35: Sample of how tasks has been handled	13

# Introduction

## Problem Statement

This project is being designed to meet all the requirements stated in the project description of MCT 382 Course. The vehicle designed is capable to fulfill and satisfy the requirements of a specified slice of the market. It should be capable of cleaning small sized rectangular pools in efficient way and average velocity.

## Motivation

The motivation to design this mobile mechatronic system comes from the fact that a small section of highly income level of customers are willing to pay money in an efficient product that can actually offers a daily care for their pools without the need of someone to break in their privacy, or having to do it their selves.

## Project Objectives

This project is being designed to follow the guidelines given by the course and meet all the requirements specified. Its general task is to provide daily care for small sized rectangular pools by removing leaves, insects or whatever that can float on the surface water of the pool.

# Design Thinking and Market Research

This project is the output of the design thinking process which included empathize; which is done though surveys and interviews of clients, define; which is done by analyzing qualitative information gathered and specifying the actual and real problem of a customer, ideate; which is done by creating many different ideas to the find a solution for the same problem, prototype; which is approached by gathering all ideas in one product and trading off between, and lastly test; which is done by going back to the clients and asks if it actually satisfies their need, if he would buy such a product, and may be how much money he would pay for that.

## Customer Segment

| ## Primary Features - Easy to Install - Compact - Average velocity - Low time consumption - Low maintenance | ## Secondary Features - Water temperature displaying - Chlorine Dispensing - Insecticides Dispensing |
| --- | --- |

Mobile robot that provides daily care for medium sized pools in villas owned by females or males who are aged between 30 -50 years old and having a relatively high income level. All of the previous data is gathered and re-arranged in a Value Proposition Canvas and Lean Canvas.

## Value Proposition Canvas

Figure 1: Value Proposition Canvas

## Lean CAnavs

Figure 2: Lean Canvas

## MarkeT Rsearch Survey

### Product concept

Product concept is to provide daily care for average sized swimming pools by cleaning the water surface from environmental contaminations as leaves and insects using a mobile robot.

Figure 3: Market Research Survey

## Analyzed Data – Survey Results

| Figure 4: Analyzed Data if customers would buy the product or not | Figure 5: Analyzed Data if customers have bought such a product before |
| --- | --- |
| Figure 6: Analyzed Data of how often would customers use this product. | Figure 7: Analyzed Data of how much would customers pay for such a product. |
| Figure 8: Analyzed Data of how many males or females would use this product. | Figure 9: Analyzed data of the ranges of ages whom would use this product. |

# Product Design and Human–Machine Interface

## Ergonomics and usability

During the phase of the product design we considered mainly the ergonomicsin the design of our product. Thus a handle is made that fits average hands’ size, placed in the exact center of the product weight, and at a suitable height for average people to avoid bending their back as much as possible, but at with a limit that it won’t cause the robot to be unbalanced.

Figure 10: Handle facilitates moving of robot

A very easy human–machine interfaceis represented for the user, an On/Off Buttonand a LED is used to indicate if the robot is in operation.

Figure 4: On/Off Button

A water detector is present between the propellers to assure that the robot is operating in a suitable water level to avoid the damage of the propellers and as a safety in case of human errors.

Figure 5: Water Sensor fixed on the robot

## Ready to Market

Aesthetics is one of our main concerns in product design, this is achieved by choosing a combination of colors that are cheerful and suitable to the environment/surrounding. Materials used are mainly polymers either PLA or acrylic, which are available in market and suitable in our application, since they have a low density and can be manufactures in a way that is suitable to come out with the robot’s parts perfectly. We consider our design a little bit innovative, since it re-used very common products in a completely different way. An average sized aquarium pump is used to perform the suction of water through a filter and bilge pumps were used as thrusters after modifying some feature and adding another.

## Form Follows Function

We assume/think that this product structure is primarily related to its intended function obviously, since a very quick look upon it from an average person can inform him/her what the product is actually doing. Propellers at the end and the boat shape at the front are the main structures that helps people to identify its function, its forward direction, and their operating environment.

Figure 6: Propellers

## Realistic Render

Here we present a realistic rendered image of the product and how it shall look at the very end of the process.

Figure 7: Realistic Rendered Image

# Design Development

## Conceptual Design

Primarily the design process was guided by minimizing the effect of drag force on the robot without creating any manufacturing complexity. Drag can be reduced by minimizing the surface area normal to the direction of travel, thus a cone/triangular shape is created at the forward direction of the vehicle, and since it’s not designed to travel backward, the normal area on the other end is rectangular.

A relatively positive buoyancydesign is determined to reduce the amount of risk of electric components to be exposed to water, although they’re well sealed, but also assure that a sufficient enough water can reach the aquarium pump, which is used as water sucker through a filter, and to maintain a suitable depth for the propellers to act upon.

Other considerations as water proof electronics and control system are bore in mind through the design process.

## Design Alternatives

There are 2 primary orientations for the thrusters which are a direct drive, where thrust is directed normal to a surface, and vector oriented thrusters, where the thrusters are aimed at various angles from the ROV to propel the ROV in various directions operating by the sum of the forces creating a resultant force in the desired direction of travel. Because of the added cost of multiple thrusters and the attempt to minimize cost this project was designed using direct thrusting techniques.

This design has 2 thrusters, for the horizontal direction moving. The initial design had a water temperature sensor and a chlorine dispenser, in addition to the already existing components, but upon pricing components, it was discovered that these would add an additional expenses to the design.

## Propsed Design

The primary design accounts horizontal travel path and rotating about the vertical axis giving a significant amount of control to the operator. This was done by implementing 2 thrusters at the rear end of the vehicle. Once the thrusters’ configuration was established, the electrical components were arranged and a frame was developed.

Figure 11:  Proposed Design

# Mechanical Design

## CAD Model

Our main interest concerning the mechanical design where the fixation of motors of the thrusters. Motors are embedded in a housing taking their cylindrical shape and bolted with studs to assure tight fixation and allow easy disassemblyin case of maintenance.

Figure 5:  Fixation of Motors Stress Analysis

## Stress ANAlysis

Since floating on the surface of water, makes almost no stress at any of the robot parts. The only part suspected to failure is the handle of the robot whenever the robot is lifted by the user. The following analysis shows that the handle can tolerate the robot’s weight.

Figure 5: Stress Analysis on the handle applying the whole weight of the robot

Figure 6: Stress analysis on a stud applying the weight of the robot

## Actuator Sizing

### MATLab Simulation

This model is primarily made as a simulation for the robot in case of controlling its velocity. But actually the available encoders were insufficient due to their shortness in operating under water. Thus an alternative idea is made as an approach to the closed loop requirement of the course.

Although this model wasn’t applicable due to the previous mentioned technical issues, it helped in actuator sizing to choose the suitable pumps.

#### Simulink (Simscape Model – Velocity Control)

Set point is compared to the signal coming from the system, and the error is fed to the PID controller which we tune its parameters until we reached the nearest point around the set point, as shown in figure (6).

Figure (8) shows the output of the PID fed to the motor driver which controls the motor using the PWM technique, while the motor shaft rotates, figure (9) shows the centrifugal pump being drove by the motor and from its flow rate, velocity outlet is obtained using the following equation:since q = Av, where q is the pump flow rate, A is the cross sectional area of the propeller enclosure, and v is the velocity of the outlet. Then, thrust of thrusters is obtained using:T = ρq (v2 – v1) where ρ is the density of water, v1 is the velocity of the vehicle, and v2 is the outlet velocity of pump while drag force is calculated using:D = T = 1/2 x CD x ρ x Anormal x v12D is the Drag acting against the vehicle, CD is the coefficient of Drag. Anormal is the area normal to the moving direction of vehicle. Thus, calculating the resultant thrust acting on vehicle.

Figure 12: Overall Model – Velocity Control

Figure 13: System

Figure 14: DC Motor & Driver

Figure 15: Load & sensing

##### Results

Figure 16: Pump flow rate			Figure 17: Vehicle Velocity

#### Simplified Model

Here is another model presenting the vehicle velocity using the previously used equations.

Drag Force against vehicle: D = T = 12 x CD x ρ x Anormal x v12

Propulsive or Thrust Force: T = ρq (v2 – v1)

Figure 18: Simplified Model

### Results

c

Figure 19: Vehicle Velocity (Simplified Model)			Figure 20: Thrust acting on vehicle (Simplified Model)

### Hand Calculations

Forces acting on vehicle horizontally are the thrust forces exerted by the thrusters to move the vehicle forward against drag force exerted by water in the opposite direction of movement, and forces acting on vehicle vertically are buoyancy forcesagainst the vehicle weight.

#### Parameters

Maximum velocity of vehicle = 0.2m/s, (v1)

Mass = 1 kg, (m)

Volume of fluid displaced = 3.54 l,(vf)

Flow rate = 3634 l/h, (q)

Density = 1000 kg/m3,(ρ)or (ρf)

Enclosure Diameter of propeller = 79.812 mm, (Den)

Anormal triangle = 5.24 x 10-3  m2

Anormal rectangle = 4.08 x 10-3  m2

Anormal cylinder = 5.514 x 10-3  m2

g = 9.8 m/s2

#### Horizontal Forces

Drag Force against vehicle: D = T = 12 x CD x ρ x Anormal x v12	Eq. (1)

Where;

D is the Drag acting against the vehicle

ρ is the density

CD is the coefficient of Drag

Anormal is the area normal to the moving direction of vehicle

v1 is the velocity of the vehicle

Known that:

CDrect= 1.05, CDtri= 0.5, CDcyl= 1.15

Substituting in Eq. (1): T = 0.32 N, resistive force needed to overcome by the thrusters.

Propulsive or Thrust Force: T = ρq (v2 – v1)		Eq. (2)

Where,

v2is the outlet velocity of pump

Solving Eq. (2) for v2:  v2 = 0.37 m/s

Since, q = Av = π(Den)24		Eq. (3)

Where,

Cross Sectional Area of Jet = A, note that the enclosure diameter is assumed to be the diameter of a jet to simplify calculations

Velocity outlet = v

Substituting in Eq. (3): q = 1.85 m3/s = 111 l/min, flow rate required to move the vehicle with a maximum velocity of 0.2 m/s

Since primarily design has 2 thrusters/pumps, each thruster requires a pump of 55.5 l/min.

#### Selected Pump/Actuator

A bilge pumps of 63 l/min flow rate is selected. Which means qnew = 3780 l/h = 0.00105 m3/ for the whole vehicle to achieve the maximum speed, meaning that each pump/thruster requires 2x10-3m3/s

Thus solving Eq. (3) for v using qnew:  v = vnew2= 0.42 m/s

Substituting in Eq. (2): Tnew = 0.462 N

## Path Generation

Using MATLab, this model shown in figure (15) simulated successfully the control of the vehicle yaw angle using an IMU. Using the famous equation that relates the torque to the angular acceleration: T = Jθ’’, and by integrating the angular acceleration about the z-axis twice, we obtain the yaw angle, which is compared to the set point and the error is fed to the PID controller of tuned parameters Kp = 10.15, Ki = 20.19, Kd = 0.446. These parameters were obtained by Zeigler – Nicholas Method. MATLab auto tuning didn’t work, since system couldn’t be linearized. PID output is fed directly to one of the thrusters and its sign is inversed and fed to the other. Thus we obtain one of the thrusters rotating in a speed higher than the other, until vehicle reaches the nearest point around the set point.

Figure (16) shows how the thrust of each thruster is calculated using the continuity equation, thus by creating a look up table between the pump flow rate and current from the pump data sheet, pump flow rate is obtained and since q = Av, where q is the pump flow rate, A is the cross sectional area of the propeller enclosure, and v is the velocity of the outlet, velocity is obtained. And since the propulsive/Thrust force: T = ρq (v2 – v1) where ρ is the density of water, v1 is the velocity of the vehicle, and v2 is the outlet velocity of pump, thus Thrust of each propeller is calculated.

In Figure (17), we calculated the Drag force resisting the thrust resulted from the thrusters using the following equation: D = T = 1/2 x CD x ρ x Anormal x v12D is the Drag acting against the vehicle, CD is the coefficient of Drag. Anormal is the area normal to the moving direction of vehicle.

### Simulink Model

Figure 21: Path Generation

Figure 22: Thrusterx

Figure 23: Drag Forcex

### Results

| Figure 24: Angle Scope | Figure 25: Displacement Scope |
| --- | --- |

## Stability

Stability is achieved by calculating the amount of volume required to be immersed versus the weight of the vehicle.

#### Vertical Forces

Buoyancy Force: FB = ρf x vf x g		Eq. (4)

Substituting in Eq. (4): FB = 34.692 N

To achieve equilibrium:

Since ΣF = 0,

Then FB = Weight of the vehicle

34.692 = mmax x 9.8

Then, mmax = 3.54 kg

Robot toppling due to misalignment of center of gravity and center of buoyancy, a wide volume is left at the front of the vehicle to allow the addition of variable weight.

## ImPLemenataTion Efforts And Resutls

### CAD Model

Chassis frontal area is made in an optimum shape to decrease the drag force as much as possible, while the lateral area is ignored since our the vehicle function won’t require to go in a reverse direction.

*figure of frontal area from cad*

We faced the difficulty of immersing the filter pump underneath water while keeping most of the electric cables away as much as possible from being immersed in water.

*figure showing the immersed volume and the non—immersed volume*

Also, the fixation of thruster onto the base through the foam made a difficulty that was solved by trying our best to cut the foam in regular curved shapes by hand, since laser cut wasn’t able to do so because of heat that will melts the foam polymer based.

*figure of real foam being cut and fixed into the base and the thruster*

### Actuator Sizing

We faced a difficulty in actuator sizing, since the usage of thrusters under this scale is not commonly used. The smallest thrusters size available in market are too big for our application, according to that an alternative is made to create a suitable thruster for this product. A bilge pump is used as a thruster by removing its impeller and impeller housing and replacing it by a small suitable available boat propeller which is designed mainly for under water applications.

Figure 26: Turning a bilge pump into a thruster

Thus, instead of sizing a motor for a thruster, a pump sizing was made instead. In addition to that, the available pump data sheet didn’t provide enough information about the motor and system identification wasn’t sufficient for some technical issues.

Accordingly, MATLab didn’t provide us any logical results although our attempts were many and continuously with the help of our teacher assistants.

### Manufacturing

We’ve used 3D-Printing in most of our manufacturing processes. Although we tried our best to avoid the common random workshops available inside and around the campus, since they’re expensive, when speaking about the workshops around the campus, or deliver very low quality of products, when speaking about the workshops of the campus, we faced the difficulty of splitting each part into too many subparts since all the available beds of the 3D-printers were too small.

Thus, more time was consumed to integrate each part, while creating a male-female assembly technique to facilitate the integration process.

### Stability

Here we had the issue of trying to achieve the exact positive buoyancy of the vehicle that allows the filter pump to be immersed while keeping the electric cables away from being immersed as much as possible.

# Electrical Design

## System Interconnection Diagram

Figure 27: SID

## Electrical Schematics

AC source is used to provide the sufficient amount of power for the robot, and conversions to 5V and 12V DC is made to meet the specs of other components.

AC source is mainly used to provide the required amount of power without any restrictions, and to avoid the heavy weight of the robot due to batteries. Also, although our application is mobile it only works in a fixed known area (swimming pool) thus AC source is the most suitable for our case.

ATmega328 (8-bit AVR Microcontroller with 4/8/16/32K Bytes In-System Programmable Flash) is used to be the only controller of the robot. It has been chosen mainly since it has enough number of digital, analogue, and PWM pins for the robot usage. In addition to, its high performance, low power consumption and simplicity.

## Wire Sizing

Our application required a compact cable of about 18 wires going out of the vehicle and into the control unit. The available compact cables have identical wires; all wires are capable of carrying equal amounts of current. Since the thrusters are the most current consuming of about 3.2 A per Thruster.  Thus 24 AWG wires are used to meet the requirements of thrusters’ current.

Figure 28: 24 AWG

## Circuit Schematics

Figure 29: ATmega328 Stand Alone Schematic Circuit

## Circuit Layouts

Figure 30:  (a) ATmega328PU Stand Alone Layout Circuit

(b) ATmega328PU Stand Alone PCB

Figure 31:  Integrated Circuit Schematic

## Power Sizing

| Calculations of each component consumption at its maximum draw of current. Components; Current (Ampere); Voltage; Power (Watt). Motors; 4.000; 12; 48.00. Ultrasonic; 0.015; 5; 0.08. Water Sensor; 0.020; 5; 0.10. IMU; 0.004; 5; 0.02. Filter Pump; 0.136; 220; 30.00. Total Current Dissipated; 4.2; Total Power; 78.19. | Calculations of current losses through the whole circuit. Components; Efficiency; Power Loss(Watt). AC - DC Converter; 80%; 15.64. DC Chopper; 96%; 3.13. Wires; 6.00. Total Power Loss; 24.8. Total Power Consumed; 103. |
| --- | --- |

## Safety

Since AC is used to power this robot, high level of safety is considered.Ready sealed products are only present in water to avoid any unexperienced mistakes made that would result in any huge accidents. High quality isolated wires were used. All Electronic components are separated in a water proof box out of water (Control Unit) that also allowed easy heat dissipation due to great stepping down of voltage.Also. A Solid State Relay with an optocoupler is used to prevent high voltages from affecting the system receiving the signal.

Components safety were achieved by using a fuse on the motor drivers and on the AC to DC converter. A circuit breaker would be better but much expensive.

Also awater sensor is used to assure that even if the start button is pressed but the robot isn’t placed in a suitable level of water, it won’t operate.

## Implementation Efforts and Results

The thrusters together consumed about 6.5A, the available inexpensive motor drivers can tolerate about 4A. Thus, two inexpensive motor drivers were cheaper than the only available motor driver that tolerate 30A, which we used.

# Software Design

Main concept of the software design is that the dimensions of the rectangular average sized swimming pool will be previously known and the robot will be pre-programmed before it’s handled to customer. Thus just by changing the width and the length and re-burning the code to the microcontroller, the robot autonomously will be able to calculate the number of turns required to clean the pool and return back to the start point, thus being ready for another trip through the swimming pool.

Figure 32: Path of Vehicle along a pool

The PID controls the PWM of the thrusters according to the IMU readings compared to the set point fed to the microcontroller. The PID parameters are tuned using Zeigler-Nicholas method, and aKp = 10.15, Ki = 20.19, and Kd =-0.446 are obtained.

Figure 33: Flowchart

## IMplementation of efforts anD REsults

We had an issue concerning the IMU calibration and noisy signals, but that was overcame using a Kalman filter.

# Project Plan

We’ve followed the VDI 2206 standard which describes the logical sequence of important sub steps in the development of a mechatronic system.

Figure 34: VDI 2206

## Project Timeline

| Project Timeline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tasks / Weeks | Week #1 | Week #2 | Week #3 | Week #4 | Week #5 | Week #6 | Week #7 | Week #8 | Week #9 | Week #10 | Week #11 | Week #12 | Week #13 | Week #14 |
| Design Thinking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stress Analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Manufacturing Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Manufacturing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hardware System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full System Integration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Paper Work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Manufacturing Timeline

| Manufacturing Timeline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parts | Thurs. 11/4 | Fri. 12/4 | Sat. 13/4 | Sun. 14/4 | Mon. 5/4 | Tues. 16/4 | Wed. 17/4 | Thurs. 18/4 | Fri. 19/4 | Sat. 20/4 | Sun. 21/4 | Mon. 22/4 | Tues. 23/4 | Wed. 24/4 |
| Bilge Pumps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Propellers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chassis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Base |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower Thruster holder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Upper Thruster holder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Floating Bed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter Pump |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Assembly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Here’s a sample of how our project tasks are scheduled.

Figure 35: Sample of how tasks has been handled

# Integration Plan

## Componenets Integration

Most components were bought ready for modular integration except the microcontroller. A standalone circuit for the ATmega328 is made and manufactured from scratch. Mainly by integrating a 16MHz oscillator with the ATmega328, a burner, and a push button for the chip reset.

## Modular Integration

Motor driver module, buck converter modules, AC to DC converter, alongside with the water sensor, ultrasonic, and IMU modules are integrated together with the microcontroller performing the whole circuit fully functioning while creating the suitable algorithm for the system control.Software design is made considering the PID controller and the PID parameters, integrated with the electrical system

## Saptial Integration

Mechanical and electrical systems are integrated together forming a fully functioning system. Thrusters are brought within their housings and fixed into the base, while ultrasonic is brought to its housing and fixed into the chassis. IMU and water sensor were fixed in a suitable to achieve their function accordingly. Other modules as the AC to DC converters, buck converter, motor driver, solid state relay and the standalone PCB of the microcontroller is fixed to the control unit box. Cables inside the control unit and the inside the vehicle is managed properly.

| Full System Integration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tasks/Days | 11/5 | 12/5 | 13/5 | 14/5 | 15/5 | 16/5 | 17/5 | 18/5 |
| Components Integration | ATmega3282P standalone circuit Schematic | ATmega3282P standalone circuit Layout | ATmega3282P standalone PCB Manufacturing |  |  |  |  |  |
| Modular Integration | AC to DC Converter - Buck Converter | Motor Driver Modules | Ultrasonic and water sensor | IMU |  |  |  |  |
| Spatial Integration |  |  |  | Fixing components to base | Fixing components to control unit | Cable Management | Algorithm creation |  |

## Integration efforts and results

Some difficulties were faced during integration, one of the most important issues is the holes made for the bolts to fasten different parts together. Holes made in base resulted in water entering the base when vehicle is placed in water, thus the vehicle sank. As a solution for that, silicon is used to assure no holes will allow water into the vehicle. The same issue was faced for the cable being inserted into the base, leaving clearance that was also avoided using silicon.

*figures of real image of silicon sealing the base*

# Test Plan

## System Verification

Thrusters, ultrasonic, water sensor, and filter pump responded correctly to the microcontroller signals and robot operated as expected. IMU had some issues concerning its calibration and noisy signals, but that is overcame using a Kalman filter.

## System Validation

Robot functioned as good as a prototype would function to prove the concept of a whole newly idea. Robot cleaned an area of about 2m2 in less than 30 seconds.

| Testing Plan |  |  |  |
| --- | --- | --- | --- |
| Tasks/Days | 20/4 | 21/4 | 22/4 |
| System Verification | Vehicle Velocity and vehicle yaw in respond of the PID Controller |  |  |
| System Validation |  | Sufficient cleaning in average time | Obstacle avoidance and low maintenance |

## Testing efforts and results

During verification, some issues have appeared like the bolts and the nuts which experienced corrosion that was solved using a transparent nail paint which was sufficient enough to preventing corrosion.

Some other difficulties were faced as the 4A motor driver which we thought would be sufficient enough for the thrusters which we calculated the required current that resulted to be 2A each, but the motor driver seemed to be supplying one of the thrusters as much as it demands without considering the other, that resulted in a thruster working correctly while the other doesn’t. This issue was solved by using 2 separate dual channel motor drivers which supplies each thruster with whatever amount of current it demands and cheaper than buying the available motor driver which can tolerate till 30A and but very expensive compared with the solution we achieved instead.

# Bill of Materials and Cost

| Parts | Quantity | Material | Manufacturing Operation | Cost (L.E) |
| --- | --- | --- | --- | --- |
| Upper Thruster holder | 2 | PLA | 3D-Prinitng | 34 |
| Lower Thruster holder | 2 | PLA | 3D-Prinitng | 25 |
| Base | 1 | PLA | 3D-Prinitng | 417 |
| Chassis | 1 | PLA | 3D-Prinitng | 290 |
| Thrusters | 2 | - | - | 900 |
| Filter pump | 1 | - | - | 250 |
| Water Detection Sensor | 2 | - | - | 156 |
| SSR - 25A (Relay) | 1 | - | - | 65 |
| Start UP Button | 1 | - | - | 3 |
| MPU 6050 (IMU) | 1 | - | - | 70 |
| USB to Serial | 1 | - | - | 40 |
| Sealed wires | 12 | - | - | 192 |
| PCB | 1 | - | - | 60 |
| Bolts M3 | 18 | - | - | 18 |
| Nuts M3 | 42 | - | - | 42 |
| filter breathing tube | 1 | PVC | - | 2 |
| Foam | 1 | Polyurethane | - | 84 |
| Ultrasonic Sesnors | 1 | - | - | 35 |
| Heat shrink | 1 | - | - | 10 |
| H-Bridge | 1 | - | - | 80 |
| Thin Human | 6 | PLA | 3D-Printing | 40 |
| Fuses | 1 | - | - | 5 |
| 26 AWG (Sealed Wires) | 1 | - | - | 150 |
| Epoxy | 1 | - | - | 51 |
| cloroform | 1 | - | - | 15 |
|  |  |  | Total Budget | 3034 |

## Supply CHAin Efforts and Results

A Low Budget criteria is mainly used for this project that resulted in a project that costed around 3000 L.E as shown in the previous figure.

### Manufacturing

We tried our best to avoid the common random workshops available inside and around the campus, since they’re expensive, when speaking about the workshops around the campus, or deliver very low quality of products, when speaking about the workshops of the campus, thus we have used 3D-printing since it allows the control of each part density and printed most of the parts on our previously made3D-Printer which allowed us to save about 25% of the cost.

### Electrical components

We’ve faced some difficulties finding bilge pumps in Cairo in addition to their highly cost, but after many researches we’ve found them available with the some random people, and they were bought second hand thuswe saved about 800L.E. 2 dual channel motor drivers tolerating 4A each were way cheaper than the available motor driver that tolerated till 30A, which was over kill for our application. In addition to that we had these motor drivers already.

# Conclusion

This project is being designed to follow the guidelines given by MCT382 course and to meet all the requirements specified. Its general task is to provide daily care for small sized rectangular pools by removing leaves, insects or whatever that can float on the surface water of the pool. It’s mainly for medium sized pools in villas owned by females or males who are aged between 30 -50 years old and having a relatively high income level. Human –Machine Interface is highly considered by providing a very easy user interface which all it requires is pushing a button.

Primarily the design process was guided by minimizing the effect of drag force on the robot without creating any manufacturing complexity. A relatively positive buoyancy design is determined to reduce the amount of risk of electric components to be exposed to water, although they’re well sealed. Water proof electronics and control system are bore in mind through the design process.

Robot toppling and stability of robot were highly considered in calculations and during design. The creation of small thruster from a pump was overcame successfully. Exact positive buoyancy is achieved by calculations which allowed us to immerse the filter pump while keeping the electric cables away from the water.

A separate control unit on the top side away from the pool has enhanced many issues as safety and solved many issues as the heavy weight of the vehicles due to the AC to DC Converter and other heavy modules. A 24AWG wire is used that was sufficient for our application and integrated the many wires required in compact thin cable.

ATmega328p is used as the microcontroller for our vehicle, its standalone PCB is made and successfully worked efficiently. According to calculations our robot consumes 103W. Electrical safety is considered by using fuses before components that re though to dissipate much current as thrusters, and a built in fuse in an AC to DC converter is already protecting it. Also, an optocoupler solid state relay is used, a ready water resistant modules and a water sensor module that protects thrusters from common human mistakes is used.

Software design is made to achieve the autonomous vehicle meant for this application by feeding the dimensions of a rectangular pool to the robot, and robot is programmed to clean the whole surface of the pool and return to its home point when finished, just by pushing a button. That is achieved under the control of a PID which is fed the error through an IMU that returns the yaw angle and is compared by the set point.

Our project plan is following the VDI 2206 standard. Project timeline, manufactured timeline, integration and testing plan were successfully followed due to logical previous planning. We tried our best to create a low budget project, which we consider ourselves have partially achieved that, since the whole budget is around 3000L.E.

# Appendix

#include "I2Cdev.h"

#include "MPU6050_6Axis_MotionApps20.h"

#include "Wire.h"

//Pool Dimensions

#define Length 700 //in cm - Pre-programmed for every pool

#define Width 500  //in cm - Pre-programmed for every pool

#define PCleaner_Width 18.5 //in cm

#define PCleaner_Length 27.1 //in cm

//Thrusters - HBridge

#define EnA  6

#define In1  7

#define In2  8

#define In3  11

#define In4  12

#define EnB  10

//Filter - SSR

#define SSR 5

//IMU - MPU6050

#define INTERRUPT_PIN 2

//Ultrasonic

#define pingPin A3

#define echoPin A2

//Water Sensor

#define Grove_Water_Sensor 3

//ON - LED

#define ON 13

//Start Button

#define Start 4

//Variables

//IMU

MPU6050 mpu;

bool dmpReady = false;  // set true if DMP init was successful

uint8_t mpuIntStatus;   // holds actual interrupt status byte from MPU

uint8_t devStatus;      // return status after each device operation (0 = success, !0 = error)

uint16_t packetSize;    // expected DMP packet size (default is 42 bytes)

uint16_t fifoCount;     // count of all bytes currently in FIFO

uint8_t fifoBuffer[64]; // FIFO storage buffer

Quaternion q;           // [w, x, y, z]         quaternion container

VectorFloat gravity;    // [x, y, z]            gravity vector

float ypr[3];           // [yaw, pitch, roll]   yaw/pitch/roll container and gravity vector

float yaw;

volatile bool mpuInterrupt = false; //for interrupt pin status

void dmpDataReady() {

mpuInterrupt = true;

}

void setup()

{

Serial.begin(9600); // Starting Serial Terminal

//Thrusters - HBridge

pinMode (In1, OUTPUT);

pinMode (In2, OUTPUT);

pinMode (EnA, OUTPUT);

pinMode (In3, OUTPUT);

pinMode (In4, OUTPUT);

pinMode (EnB, OUTPUT);

//Filter - SSR

pinMode(SSR, OUTPUT);

//IMU - MPU6050

//I2C

Wire.begin(); //Initiate the Wire library and join the I2C bus as a master

Wire.setClock(400000); // 400kHz I2C clock

// MPU 6050 device initialize

mpu.initialize();

pinMode(INTERRUPT_PIN, INPUT);

devStatus = mpu.dmpInitialize(); //function returns zero if it worked

// supply your own gyro offsets here, scaled for min sensitivity - Default Values

mpu.setXGyroOffset(220);

mpu.setYGyroOffset(76);

mpu.setZGyroOffset(-85);

mpu.setZAccelOffset(1788); // 1688 factory default for my test chip

if (devStatus == 0)

{

mpu.setDMPEnabled(true);

// enable Arduino interrupt detection

attachInterrupt(digitalPinToInterrupt(INTERRUPT_PIN), dmpDataReady, RISING); //trigger when the pin goes from low to high

mpuIntStatus = mpu.getIntStatus();

dmpReady = true;

// get expected DMP packet size for later comparison

packetSize = mpu.dmpGetFIFOPacketSize();

}

//Ultrasonic

pinMode(echoPin, INPUT);

pinMode(pingPin, OUTPUT);

//Water Sensor

pinMode(Grove_Water_Sensor, INPUT);

//ON - LED

pinMode(ON, OUTPUT);

//Start Button

pinMode(Start, INPUT);

}

void loop()

{

int water = digitalRead(Grove_Water_Sensor);//LOW when water is detected

//int start = digitalRead(Start);//LOW when button is pressed

int start = LOW;

int distance = Distance();

while (!water && !start)

{

if (distance > 20) // if ( mode && distance > 20)

{

VehicleForward();

digitalWrite(SSR, HIGH);//Filter Energized

delay (1000);

}

if (distance < 20)

{

ThursterSTOP(1);//Stop Right Thruster

ThursterSTOP(2);//Stop Left Thruster

int prev_Angle = Angle();

Thruster(2, 2, 150); //Energize Left Thruster

while (prev_Angle < Angle()+180); //Wait till required angle is reached -PID should be present in here

ThursterSTOP(2);//Stop Left Thruster

}

VehicleForward();

}

}

//////////////////////////////////////////////////////////////////////////////////////////////////////////

///////////////                         DEFINITIONS.H - Starts                          //////////////////

/////////////////////////////////////////////////////////////////////////////////////////////////////////

//for UltraSonic

long microsecondsToCentimeters(long microseconds)

{

return microseconds / 29 / 2;

}

float Distance()

{

digitalWrite(pingPin, LOW);

delayMicroseconds(2);

digitalWrite(pingPin, HIGH);

delayMicroseconds(10);

digitalWrite(pingPin, LOW);

int duration = pulseIn(echoPin, HIGH);

int cm = microsecondsToCentimeters(duration);

}

//for Thrusters

void ThursterSTOP(int thruster)

{

if (thruster == 1)

{

digitalWrite (In1, LOW);

digitalWrite (In2, LOW);

analogWrite (EnA, 0);

}

if (thruster == 2)

{

digitalWrite (In3, LOW);

digitalWrite (In4, LOW);

analogWrite (EnB, 0);

}

}

void Thruster(int thruster, char dir , int pwm)

{

//Thruster: 1 (Right) or 2 (Left)

//dir:  1 (Clockwise) or 2 (Counter Clockwise)

//pwm: 0 to 255

if (thruster == 1, dir == 2)

{

digitalWrite (In1, HIGH);

digitalWrite (In2, LOW);

analogWrite (EnA, pwm);

}

else if (thruster == 2, dir == 2)

{

digitalWrite (In3, HIGH);

digitalWrite (In4, LOW);

analogWrite (EnB, pwm);

}

else if (thruster == 1, dir == 1)

{

digitalWrite (In1, LOW);

digitalWrite (In2, HIGH);

analogWrite (EnA, pwm);

}

else if (thruster == 2, dir == 1)

{

digitalWrite (In3, LOW);

digitalWrite (In4, HIGH);

analogWrite (EnB, pwm);

}

}

void VehicleForward()

{

Thruster(1, 2 , 255);

Thruster(2, 2 , 255);

}

//for IMU

float Angle()

{

if (!dmpReady) return;// if programming failed, don't try to do anything

while (!mpuInterrupt && fifoCount < packetSize) // wait for MPU interrupt or extra packet(s) available

{

if (mpuInterrupt && fifoCount < packetSize)

fifoCount = mpu.getFIFOCount();

}

mpuInterrupt = false; //clear interrupt flag

mpuIntStatus = mpu.getIntStatus(); // get INT_STATUS byte

fifoCount = mpu.getFIFOCount();//// get current FIFO count

// check for overflow (this should never happen unless our code is too inefficient)

if (mpuIntStatus & _BV(MPU6050_INTERRUPT_DMP_INT_BIT))

{

while (fifoCount < packetSize) // wait for correct available data length

fifoCount = mpu.getFIFOCount();

mpu.getFIFOBytes(fifoBuffer, packetSize);  // read a packet from FIFO

// display Euler angles in degrees

mpu.dmpGetQuaternion(&q, fifoBuffer);

mpu.dmpGetGravity(&gravity, &q);

mpu.dmpGetYawPitchRoll(ypr, &q, &gravity);

yaw = ypr[0] * 180/M_PI;

return yaw;

}

}

//////////////////////////////////////////////////////////////////////////////////////////////////////////

///////////////                         DEFINITIONS.H - ENDS                           //////////////////

/////////////////////////////////////////////////////////////////////////////////////////////////////////